| Motor | KV | Stator | Max thrust | Hover current | Max current | Efficiency | Rating |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EMAX RS2205 Racing motor | 2300 KV | 22×5 mm | ~800 g | ~12 A | ~28 A | ~6 g/W | Under thrust |
| T-Motor F60 Pro IV Cinematic / survey | 1750 KV | 22×6 mm | ~1200 g | ~8 A | ~32 A | ~9 g/W | Recommended |
| BrotherHobby Avenger 2806.5 Long-range build | 1300 KV | 28×6.5 mm | ~1500 g | ~7 A | ~35 A | ~10 g/W | Heavy frame needed |
| SunnySky X2216 Budget survey | 1250 KV | 22×16 mm | ~950 g | ~9 A | ~22 A | ~7 g/W | Lower efficiency |

| ESC | Continuous current | Burst current | Protocol | BEC | Telemetry | Rating |
| --- | --- | --- | --- | --- | --- | --- |
| Hobbywing XRotor 40A | 40 A | 50 A (10s) | DSHOT300/600 | No | Yes | Good, bulkier |
| BLHeli_32 45A (4-in-1) | 45 A | 55 A (10s) | DSHOT150/300/600 | 5V 2A | Yes (ESC temp) | Recommended |
| AM32 35A (4-in-1) | 35 A | 40 A (10s) | DSHOT300/600 | No | Yes | Tight margin |

Motor: T-Motor F 60 Pro IV has a peak efficiency of over 83% in the 3–10 A current range, which is precisely where a hovering scout drone operates — meaning you're not wasting battery on heat during the 30-minute scan mission. Low-KV motors achieve superior efficiency when spinning large propellers, with the increased torque enabling larger prop diameters that produce more thrust per watt than small props. Stuff This is why 920 KV + 9.5" prop beats a higher-KV motor on a smaller prop for endurance.

ESC: BLHeli_32 — chosen as primary because BLHeli_32's ESC telemetry (RPM, temperature, current) feeds directly into the ROS2 pipeline via MAVLink, enabling real-time motor health monitoring during the 30-minute scan mission. The 30 A rating gives a 1.67× safety margin over the motor's 18 A continuous.