| Propeller | Size | Material | Max thrust (4S) | Max current | Efficiency | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| T-Motor T5143S Official match | 5.1×4.3" | Carbon nylon | ~820 g | ~22 A @ 100% | Best efficiency for 1750KV — low pitch suits survey hover | Recommended |
| T-Motor T5146 Higher pitch | 5.1×4.6" | Carbon nylon | ~870 g | ~25 A @ 100% | More thrust but higher current draw | Alternative |
| T-Motor T5147 Racing tune | 5.1×4.7" | Carbon nylon | ~900 g | ~28 A @ 100% | High current, reduced flight time | Too aggressive for survey |

The T5143S has the lowest pitch of the tested props, keeping current draw lower at hover throttle (~40–50%). For a survey/cinematic mission the drone spends most of its time hovering or cruising slowly — low pitch = less wasted energy = longer flight time. The 3-blade design balances efficiency and vibration smoothness, important for stable camera footage and YOLO detection.

| Component | Model | Purpose | Key specs |
| --- | --- | --- | --- |
| Flight controller | Holybro Pixhawk 6C Mini | Stabilisation, autonomous flight, ROS2 bridge via MAVROS | STM32H743, 38×38 mm, dual IMU, PX4 native |
| GPS module | Holybro M10 Mini | Position hold, survivor geotagging | u-blox M10, GPS+GLONASS+BeiDou+Galileo, 1.5 m CEP, UART |
| Telemetry module | mRo SiK 915 MHz | MAVLink comms to ground station | 915 MHz ISM, 300 mW, 3 km range, bi-directional UART |
| Onboard computer | Raspberry Pi 5 (4GB) + Hailo-8L HAT | YOLOv8 human detection, ROS2 node execution | Cortex-A76 @ 2.4 GHz, 4GB LPDDR4X, 13 TOPS AI, CSI port |
| Power distribution | Holybro Kakute H7 Stack | Power distribution, BEC, current sensing | 45A per ESC, 5V/3A BEC, integrated current sensor, 30.5 mm stack |